Simulate VLAN Configuration in Packet Tracer

Aim: To understand and configure VLANs for network segmentation and to observe communication between devices within and across VLANs.

Steps to Simulate VLAN Configuration:

1. Open Cisco Packet Tracer:
Launch the Cisco Packet Tracer application.

2. Add Network Devices:
Add a Switch:
From the Network Devices section, drag and drop a Switch (e.g., 2960 model) onto the workspace.
Add PCs:
From the End Devices section, drag and drop 4 PCs onto the workspace.
Add a Router:
Drag and drop a Router (e.g., 1941 model) from Network Devices to the workspace for inter-VLAN communication.

3. Cable the Network
Use a Copper Straight-Through Cable to connect:
PC1 to FastEthernet0/1 of the switch.
PC2 to FastEthernet0/2 of the switch.
PC3 to FastEthernet0/3 of the switch.
PC4 to FastEthernet0/4 of the switch.

Switch to the Router's GigabitEthernet0/0 interface using a Copper Straight-Through Cable.

4. VLAN Creation on the Switch
Access the Switch CLI:
Click on the switch and go to the CLI tab.

Enter Configuration Mode:
CODE:
enable
configure terminal

Create VLANs:
Create VLAN 10 and VLAN 20:
CODE:
vlan 10
name Sales
exit
vlan 20
name Marketing
exit

Assign VLANs to Ports:
Assign FastEthernet0/1 and FastEthernet0/2 to VLAN 10:

CODE:
interface fastethernet0/1
switchport mode access
switchport access vlan 10
exit
interface fastethernet0/2
switchport mode access
switchport access vlan 10
exit

Assign FastEthernet0/3 and FastEthernet0/4 to VLAN 20:

CODE:
interface fastethernet0/3
switchport mode access
switchport access vlan 20
exit
interface fastethernet0/4
switchport mode access
switchport access vlan 20
exit

Verify VLAN Configuration:
CODE:
show vlan brief

5. Configure the Router for Inter-VLAN Communication
Assign Subinterfaces:
Access the router CLI and configure GigabitEthernet0/0:

CODE:
enable
configure terminal
interface gigabitethernet0/0.10
encapsulation dot1Q 10
ip address 192.168.10.1 255.255.255.0
exit
interface gigabitethernet0/0.20
encapsulation dot1Q 20
ip address 192.168.20.1 255.255.255.0
exit

Activate the Router Interface:

CODE:
interface gigabitethernet0/0
no shutdown
exit

6. Configure IP Addresses on PCs

Assign static IP addresses to each PC as follows:
PC0: 192.168.10.2, Subnet Mask: 255.255.255.0, Gateway: 192.168.10.1

PC1: 192.168.10.3, Subnet Mask: 255.255.255.0, Gateway: 192.168.10.1

PC2: 192.168.20.2, Subnet Mask: 255.255.255.0, Gateway: 192.168.20.1

PC3: 192.168.20.3, Subnet Mask: 255.255.255.0, Gateway: 192.168.20.1

On each PC:
Open the Desktop tab.
Select IP Configuration.
Input the above IP details.

7. Test Communication

Ping PCs in the Same VLAN:
From PC1, ping PC2 (successful).
From PC3, ping PC4 (successful).

Ping PCs Across VLANs:
From PC1, ping PC3 (successful).
From PC2, ping PC4 (successful).

8. Outcome:

VLANs were successfully created and configured on the switch.
Communication within the same VLAN was successful.
Inter-VLAN communication was facilitated using a router.
Students learned practical VLAN configuration and inter-VLAN routing.