Spanning Tree Protocol (STP) Simulation Guide

Objective:

Prevent network loops using Spanning Tree Protocol (STP) to ensure network stability.

Steps:

1. Create a Network with Redundant Links

· Open Cisco Packet Tracer.

· Add at least three switches to the workspace (e.g., Switch0, Switch1, and Switch2).2960-24TT

· Interconnect the switches with redundant links. For instance:

o Connect Switch0 to Switch1 with two links.

o Connect Switch1 to Switch2 with two links.

o Connect Switch0 to Switch2 with two links.

· Use crossover cables for inter-switch connections if using older switches; otherwise, use straight-through cables.

2. Enable STP on the Switches

· Configure the switches:

1. Go to the CLI (Command Line Interface) of each switch (repeat the same step for each switch).

2. Enter the following commands:

Code:

enable
configure terminal
spanning-tree vlan 1
exit

o The spanning-tree vlan 1 command enables STP for VLAN 1 (default VLAN).

· Verify STP operation:

o Use the show spanning-tree command to observe the status of STP. The output should show:

§ The Root Bridge (the switch elected as the root).

§ The ports in forwarding state.

§ The ports in blocking state (redundant links).

3. Observe STP in Action

· Once STP is enabled:

o Check which ports are blocked to prevent loops.

o The show spanning-tree command will display which interfaces are in Forwarding or Blocking states.

4. Simulate a Link Failure

· Disconnect one of the primary links between two switches. For example:

o Disconnect a link between Switch0 and Switch1.

· Observe how STP reacts:

o Use the show spanning-tree command to verify that a blocked link is now in the forwarding state, restoring network connectivity.

5. Verify Network Stability

· Use ping to ensure connectivity between devices on the network remains intact during the simulation.

· Verify that STP successfully prevented any loops.

Outcome:

By completing this simulation, you will:

· Understand how STP blocks redundant links to prevent network loops.

· See STP’s role in automatically reactivating links to maintain network stability when a primary link fails.

· Gain practical knowledge of configuring and monitoring STP in a network environment.